# Final Project Report

Project Title: Smart Attendance System using RFID & Face Recognition

Project Description:

- The Smart Attendance System is a dual-authentication security and attendance management device designed to ensure reliable identity verification in workplaces, classrooms, and secure zones.

- The system integrates RFID card scanning with face recognition technology, requiring both verifications before granting access. This approach prevents unauthorized entry and eliminates the risk of fake or proxy attendance.

- Attendance is automatically recorded in real time once authentication is successful, removing the need for manual entry.

- The device is capable of supporting multiple users by storing RFID unique IDs and corresponding face data, making it scalable.

- Its low power consumption and portability make it suitable for classrooms, offices, and restricted areas, with potential future versions supporting battery operation.

- By combining security and automation, this project demonstrates how embedded systems and IoT principles can be applied to solve real-world challenges in attendance tracking.

Component List:

| Item | Quantity | Price(tk) | Subtotal(tk) |
| --- | --- | --- | --- |
| ESP-32 CAM Development Board | 1 | 890 | 890 |
| ESP-32 Micro-USB Download Module | 1 | 220 | 220 |
| 16x2 Serial LCD (I2C) Module Display | 1 | 340 | 340 |
| RFID RC522 Card Reader Module | 1 | 179 | 179 |
| RFID NFC Smart Card 13.56 MHz | 3 | 32 | 96 |
| DC-DC Step Up Boost Converter (MT3608) | 1 | 89 | 89 |
| 18650 Li-ion Battery | 1 | 95 | 95 |
| 18650 Battery Holder – 1 Cell | 1 | 40 | 40 |
| Breadboard Full Size Bare 830 | 1 | 145 | 145 |
| Active Buzzer 5V | 1 | 18 | 18 |
| 5 mm Red LED | 1 | 5 | 5 |
| 220 Ohm Resistor | 1 | 5 | 5 |
| Jumper Wires (Male to Female, 40 pcs) | 1 | 100 | 100 |
| USB Data Cable Type-B | 1 | 180 | 180 |
| Total |  |  | 2,402 |

## Working Methodology:

The Smart Attendance System works by sequentially verifying RFID card data and face recognition results before granting access and recording attendance. The workflow is as follows:

### Step 1 – Arduino & Component Setup

• Arduino Uno is connected to all peripheral modules.
     • Each module (RFID, LCD, LEDs, buzzer) is tested individually for proper operation.

### Step 2 – RFID Verification

• The user scans their RFID card using the RC522 module.
• If the card is invalid, the system directly displays “Access Denied” and activates the red LED + buzzer.
• If valid, the process moves to face recognition.

### Step 3 – Face Recognition with ESP32-CAM

• Once a valid card is scanned, the LCD shows “Face Recognizing…”.
    • The ESP32-CAM module captures the user’s face and compares it with stored data.
    • If the face matches, the system grants access and records attendance.
    • If not, access is denied.

### Step 4 – Feedback Mechanisms

• LCD Display – Shows live authentication messages.
    • LEDs – Green LED for granted, Red LED for denied.
    • Buzzer Patterns:

- 1 beep → RFID granted

- 2 beeps → Face granted

- 3 beeps → RFID denied

- 5 beeps → Face denied

- 10-second continuous beep → Hardware/system error

### Step 5 – Power Supply & Final Assembly

• The system is powered via USB or an external 5V adapter, with optional battery-based    portability using a Li-ion battery and boost converter.
• All components are interconnected using jumper wires and mounted on a breadboard for prototyping.

.

System Flow Diagram:

## Future Implications & Improvements:

- Database Integration:
• Store attendance logs in a centralized database (e.g., Firebase, MySQL) for real-time monitoring by teachers/admins.

- Mobile & Web Application:
• Develop an app/dashboard to track attendance records remotely.
• Send notifications for successful or failed authentication attempts.

- Offline Face Recognition Storage:
• Extend ESP32-CAM to store more face data for multi-user scalability.

- Battery-Powered Portability:
• Use a rechargeable Li-ion pack with solar charging for outdoor or mobile use.

- Cloud & IoT Features:
• Connect to cloud servers for centralized data access across multiple classrooms or offices.

- Security Enhancement:
• Add fingerprint authentication or OTP-based mobile verification as a third factor for highly secure environments.